Kai — Hardware Layout & Wiring Build Sheet

Edge Offline AI Robot — hardware layout, wiring & physical setup only — rev E

## TL;DR — as-built state & current status

This document describes the hardware layout and wiring for Kai as it is actually built. All four subsystems below — mic, servos, camera and the speaker chain — are connected and running on the robot. Where the as-built wiring differs from the ideal layout, the difference is recorded in place along with what it costs.

Kai is a four-segment vertical cardboard stack: head (jaw servo at the mouth) → neck box (pan servo, Arduino Uno, fixed forward-facing Pi Cam NoIR V2) → body drum (all audio hardware) → exposed Jetson Orin Nano base (stays visible by design). Two servos only — pan (neck, D9) and jaw (head, D6) — driven by the Uno over PWM, with face-tracking data from the Jetson over USB serial.

| Subsystem | Status | Detail |
| --- | --- | --- |
| INMP441 mic → Jetson | Connected | Wired to 40-pin header on I2S2 (see section 4). Established. |
| Servos (pan + jaw) | Connected | Pan (D9) and jaw (D6) wired to the Uno. |
| Camera (Pi NoIR V2) | Connected | CSI ribbon to Jetson base, fixed forward-facing. IMX219-class sensor; it does not pan with the head. |
| Speaker → PAM8403 → 3.5mm → Jetson | Connected | Chain works end to end: Jetson → USB sound card (C-Media) → 3.5mm → PAM8403 → speakers. Kai speaks its replies through this path and the jaw is driven from the same audio. |
| Arduino Uno → Jetson (USB) | Connected | USB-A to USB-B. CH340 clone — the ch341 driver is not in the Jetson kernel and is loaded from hardware/ch341_build (see 9.2). |
| Jetson → network (Wi-Fi / Ethernet) | Connected | Serves the dashboard on port 8081 and accepts SSH. Changing which Wi-Fi network Kai joins is done over the wire — see section 10. |

## 1 — Vertical stack overview

Kai is a four-segment vertical stack. The Jetson stays exposed at the base (design requirement, logo/branding visible). The body drum holds the entire audio subsystem. The neck box holds the pan servo, the Arduino Uno and the fixed forward-facing camera. The jaw servo lives up in the head at the mouth linkage.

## 2 — Body drum placement (to scale)

Speakers fire forward through the front wall, flanking the 4-circle logo which stays intact. Mic sits rear-high, foam-mounted, with its own rear port. Amp sits rear-low with a screwdriver access hole for the POT.

Interior coordinates (mm), origin bottom-front-left. X=width, Y=depth front-to-back, Z=height.

| Component | X (L-R) | Y (F-B) | Z (bot-top) | Notes |
| --- | --- | --- | --- | --- |
| Speaker L | 5-33 | 0-15 | 0-15 | front-firing, left of logo |
| Speaker R | 67-95 | 0-15 | 0-15 | front-firing, right of logo |
| Logo cluster | 40-60 | front face | centered | 4 circles, untouched |
| INMP441 mic | ~50 | 75-79 | 30-42 | rear/high, foam mount, own port |
| PAM8403+POT | 40-65 | 74-79 | 0-12 | rear/low, POT trim hole in wall |

## 3 — Full wiring schematic

Five signal domains kept physically separate to control noise. The Uno talks to the Jetson over USB serial and drives both servos by PWM. The mic is I2S to the 40-pin header; the camera is CSI ribbon; the amp takes analog line-out from a USB sound card on the Jetson.

## 4 — INMP441 to Jetson 40-pin header (I2S2)

Exact mapping per your setup. Note VDD is 3.3V (Pin 1), not 5V — the INMP441 is a 3.3V part and 5V can damage it. L/R is tied to GND, placing the mic on the left channel of the I2S2 stereo stream.

| INMP441 pin | Jetson pin | Signal | Wire color (suggest) |
| --- | --- | --- | --- |
| VDD | Pin 1 | 3.3V power | red |
| GND | Pin 6 | Ground | black |
| SD | Pin 38 | I2S2_SDIN (data in) | green |
| SCK | Pin 12 | I2S2_SCLK (bit clock) | yellow |
| WS | Pin 35 | I2S2_FS (frame/word sel) | blue |
| L/R | Pin 9 | GND = left channel | white |

Note: this sheet covers hardware and physical setup only. Enabling the I2S pins in software (device-tree configuration and the capture/record test) is handled separately and is not part of this document.

## 5 — Arduino Uno servo wiring

Two SG90 servos only. Signal pins are suggestions — match them to your reference firmware's config. SG90 signal is 5V-logic PWM, which the Uno drives directly.

| Servo | Uno signal pin | Power | Face param → motion |
| --- | --- | --- | --- |
| Pan (neck) | D9 (PWM) | Uno 5V pin / GND (shared) | X → head left/right |
| Jaw (head) | D6 (PWM) | Uno 5V pin / GND (shared) | TTS speaking → mouth open/close |

As built, both servos take 5V from the Uno's onboard 5V pin and the Uno is powered from the Jetson's USB port — so the servos, the board and the CH340 USB chip all share one USB rail. That is why the pan channel is gated to 10 Hz and 8 degrees per command in software: larger or faster commands raise average current and can brown out the CH340, which drops /dev/ttyUSB0 mid-track. If you want faster head motion, give the servos their own 5V supply with grounds commoned to the Uno — do not raise the send rate instead.

## 6 — Power & ground scheme

| Rail / net | Feeds | Rule |
| --- | --- | --- |
| 5V — audio | PAM8403 + USB sound card | Own run from 5V src. Add 100-470uF bulk + 0.1uF at amp Vcc. |
| 5V — servo | Pan + jaw SG90 | As built: Uno 5V pin, off the Jetson USB rail, shared with the CH340 — which is why pan stays at 10 Hz / 8 deg per command. A separate 5V run is the upgrade path. Never share the amp rail. |
| 3.3V | INMP441 VDD | From Jetson Pin 1 only. Never 5V to the mic. |
| Ground | All modules | Star ground at the 5V src. No daisy-chaining amp → servo → Jetson. |
| I2S lines | INMP441 → P12/35/38 | Keep < 30cm, route away from servo PWM and speaker leads. |
| 3.5mm jack | Sound card → amp | Tip=L, Ring=R, Sleeve=GND. Matches Spk L / Spk R. |

## 7 — Pre-power checklist

- ☐  INMP441 VDD is on Pin 1 (3.3V), NOT Pin 2/4 (5V). Verify before first power-on.

- ☐  L/R pin tied to GND (Pin 9). Floating L/R gives silence or noise.

- ☐  Amp 5V and servo 5V are separate runs from the source; grounds star to one point.

- ☐  Bulk + bypass caps fitted on PAM8403 Vcc, or USB/servo noise will be audible.

- ☐  Mic foam/grommet mount in place — decouples speaker vibration through the cardboard.

- ☐  Speakers fire front (Y=0), mic ports rear (Y=79): the isolation split is intact.

- ☐  CSI ribbon has a gentle radius (no crease) and runs clear of the pan-servo PWM leads.

- ☐  3.5mm plug fully home in the USB sound card — a half-inserted plug gives one channel or a loud hum.

- ☐  Servo horns mechanically centred at 90 deg before power-on, so travel limits are set in software and not by cardboard.

## 8 — Acoustic & integration notes

- Mic isolation is the #1 risk. The always-on I2S mic shares the drum with two speakers. Foam/grommet soft-mount decouples structure-borne vibration through the cardboard — this matters more than air-path distance in a cardboard box.

- Two servos only. Pan (neck) and jaw (head). The jaw is up in the head, far from the mic, so jaw-noise-into-mic is broken by distance. Only the speakers remain as an in-drum noise source.

- Camera is fixed forward. It does not pan with the head, so the tracking reference frame stays stable while the head it drives rotates above it.

- Left-channel mic. With L/R→GND the mic data lands on the left channel of the S32_LE stereo stream; take the left channel (or downmix) in the capture code.

## 9 — Common issues & fixes

What actually goes wrong with Kai, grouped by subsystem, with the cheapest fix first. Most of these present as software faults and are not: a silent robot, a deaf robot and a robot that restarts itself are usually a rail, a plug or a driver.

9.1 — Power, boot and the robot process

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Nothing happens at power-on; no lights on the Jetson | Underpowered supply — a phone charger cannot start an Orin Nano | Use the board's own supply. A brownout at boot looks exactly like a dead board. |
| Board boots but Kai never comes up | face_track.py is launched by an @reboot cron entry running scripts/autostart.sh | Check the process is there before suspecting hardware. If it starts but every reply is silent, run loginctl enable-linger once, then reboot. |
| Kai relaunched itself mid-demo and repeated its greeting | The supervisor did its job — the process exited or crashed and autostart.sh brought it back | Not a hardware fault by itself. Read /tmp/face-servo.log for the exit code; rc=139 is a segfault and is usually audio-device contention (see 9.4). |
| ext4 errors printed on the rootfs at boot | Known: the rootfs partition mounts with recorded errors | Run e2fsck with the filesystem unmounted before trusting the board for a demo, and always shut down properly rather than cutting power. |
| Dashboard unreachable on port 8081 | Kai is not running, or the board is on another network/IP | Ping the Jetson first. The dashboard is served by face_track.py itself — no page means no process, not a web problem. If you cannot find the board's IP at all, see section 10. |

9.2 — Servos and the Arduino link

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| /dev/ttyUSB0 missing; Kai starts with no servos | The ch341 driver for the Uno's CH340 clone is not in the Jetson's kernel | sudo bash hardware/fix_usb.sh. Once only, sudo bash hardware/install_udev.sh for auto-bind on every plug. |
| Servo buzzes but does not rotate | Faulty SG90, or the 5V rail is sagging under stall current | Sweep it standalone: python3 -m servo.servo_serial --sweep. If it still only buzzes, replace the servo. |
| Head jerks, then the serial link drops mid-track | SG90 current spikes on the shared USB rail brown out the CH340 | Keep the 10 Hz / 8 deg gates in place (section 5). The real fix is a separate 5V servo supply. |
| Head tracks the wrong way | Horn or mounting is mirrored relative to the camera | Launch with --flip. Do not re-mount the servo to correct direction. |
| Jaw does not move while Kai speaks | Jaw not wired to D6, or the jaw channel is not enabled | The jaw needs the wire on D6 and --jaw at launch. |
| Servos hum continuously while idle | Something is still sending commands — the firmware detaches after 4 s of silence | A healthy idle robot goes quiet. A continuous hum means a second process is holding the port. |
| Servo strains or the linkage binds at end of travel | Mechanical range does not match the commanded angle range | Centre the horn at 90 deg mechanically, then bound travel in config/servo.py. |

9.3 — Microphone (INMP441 on I2S)

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Kai does not react to speech; the mic reads as not live | The INMP441 comes up on one boot and not the next | Press "Find the microphone again" on the dashboard first (~2 s). It re-runs the whole discovery path without a restart. |
| Mic dead or hot from first power-on | VDD wired to 5V | VDD goes to Pin 1 (3.3V) only. A part that has seen 5V is usually dead — replace it and re-check the pin. |
| Capture is pure silence, or pure noise | L/R left floating | Tie L/R to GND (Pin 9). Data then lands on the left channel, which the capture code takes. |
| Kai hears its own speakers, or picks up buzz | Structure-borne vibration through the cardboard | Foam/grommet soft-mount the mic; keep it rear-high with its own port and the speakers front-firing. |
| Kai hears, but on the wrong microphone | Selection fell back to the USB dongle when the I2S probe timed out | The "find the microphone" reply says which device it landed on. Re-seat the I2S wiring rather than living on the fallback. |
| Intermittent dropouts or clicking in captured audio | I2S run too long, or routed alongside servo PWM and speaker leads | Keep the four I2S wires under 30 cm and physically away from the PWM and speaker runs. |

9.4 — Speaker, amp and output

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Every reply is silent though the dashboard says it spoke | The C-Media card flipped its profile to S/PDIF | Kai re-asserts the analog profile on every start, so a restart is the quick fix. pactl list short sinks confirms which sink is really there. |
| Audio dies after a quiet spell; next reply clipped at the start | PulseAudio suspend-on-idle | The keep-alive in the user default.pa plus the udev rule already fix this. If it returns, check both survived the last rebuild. |
| Buzz or whine that rises with head movement | Amp and servos sharing a rail, or grounds daisy-chained | Separate 5V runs from the source, star ground at one point, fit bulk 100-470uF plus 0.1uF bypass at the amp Vcc. |
| Distortion or clipping on loud replies | PAM8403 POT set too high, or software volume above 1.0 | Trim the POT through the rear access hole first; only then adjust volume in the Settings tab. |
| Only one speaker plays | 3.5mm plug not fully seated, or a mono (TS) plug in a TRS jack | Push the plug fully home. Tip = L, ring = R, sleeve = GND. |
| Kai segfaults exactly as its first audio starts | Playback re-opened the sound card underneath a live capture stream on that same card | Happens when capture has fallen back to the C-Media dongle. Get capture back onto the I2S mic (9.3). |

9.5 — Camera

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| No camera at boot; dashboard reports NO CAMERA with a reason | CSI is probed once at boot, and creating the device node needs a root driver bind | sudo bash scripts/camera_bind_retry.sh --now, or --install once to retry at every boot. Kai picks the camera up within ~5 s. |
| A camera plugged in after boot never appears | Same CSI bind-at-boot limitation | USB webcams genuinely hot-plug; a CSI module does not. Kai running with no camera is a reported state, not a failure. |
| i2cdetect shows an empty CSI bus | The sensor is only powered for ~10 ms while the driver probes | sudo bash scripts/camera_diag.sh probes inside that window and says whether a sensor answers at all. |
| Image is upside down or sideways | Module mounted rotated in the neck box | Use --rotate 0/90/180/270 rather than remounting the camera. |
| Tracking stutters or drifts after a head move | Ribbon creased, or running beside the pan PWM leads | Give the ribbon a gentle radius and route it clear of the servo leads. |

9.6 — First aid, cheapest first

- 1. Find the microphone again — about 2 s, nothing else interrupted. For anything where Kai cannot hear.

- 2. Restart Kai — about 30 s, the conversation is lost. For anything a setting cannot reach.

- 3. Reboot the Jetson — about 90 s, the whole board goes down. For wedged kernel audio, a wedged camera stack or GPU fragmentation.

- Reaching for the bottom of that ladder first is how a five-second problem becomes ninety seconds of dead robot.

# 10 — Network setup & changing Wi-Fi

The Jetson serves the dashboard on port 8081 and accepts SSH over whatever network it is joined to. At a new venue the saved Wi-Fi will not exist, so Kai comes up with no network and the dashboard is unreachable until you put it on the local Wi-Fi. There is no keyboard or monitor on the robot by default, so the reliable path is: get a shell on the Jetson, then change the Wi-Fi from that shell.

Two ways to get that first shell when Kai is on an unknown network: (a) plug the Jetson into a monitor and keyboard, or (b) connect it to your laptop/router by Ethernet and SSH in. Once you have a shell, section 10.2 changes the Wi-Fi. Sections 10.3–10.4 cover finding the IP so you can SSH in the first place.

The Jetson runs Ubuntu (L4T). All commands below run ON the Jetson.

## 10.1 — Getting a shell: SSH from your computer

SSH is the same command on macOS and Linux (both ship an OpenSSH client in the terminal). On Windows it is built into PowerShell as well. The Jetson's Linux user is devconph, so the login is devconph@<jetson-ip>. You will be prompted for the account password on first connect (see the credential note below).

| Your computer | Where you type it | Command |
| --- | --- | --- |
| macOS | Terminal.app | ssh devconph@<jetson-ip> |
| Linux | Any terminal | ssh devconph@<jetson-ip> |
| Windows | PowerShell (built-in OpenSSH) | ssh devconph@<jetson-ip> |
| Windows (WSL) | Ubuntu/WSL shell | ssh devconph@<jetson-ip> |

Login credentials — user devconph, password devconph123. The same password is used for the sudo commands in 10.2. Change it (passwd) before any public or shared deployment.

If you already know the IP (for example the router shows it, or Kai printed it on a monitor), skip to 10.2. If you do not know the IP, find it with mDNS (10.3) or by scanning the network (10.4).

## 10.2 — Changing the Wi-Fi network (on the Jetson, Ubuntu)

Ubuntu on the Jetson manages Wi-Fi with NetworkManager, driven from the terminal by nmcli. This works headless over SSH or on a directly attached monitor. Run these on the Jetson.

- Confirm Wi-Fi is on and list nearby networks:

nmcli radio wifi on

nmcli device wifi rescan

nmcli device wifi list

- Join a network (it is saved and auto-rejoined on later boots):

sudo nmcli device wifi connect "<SSID>" password "<passphrase>"

Wrap the SSID and password in quotes if they contain spaces or symbols. For a hidden network add hidden yes.

- Verify it connected and read back the new IP:

nmcli -t -f ACTIVE,SSID device wifi | grep '^yes'

ip addr show wlan0 | grep 'inet '

hostname -I

That new wlan0 address is where the dashboard now lives — browse to http://<that-ip>:8081 from any device on the same Wi-Fi. To forget an old venue's network so it does not linger: sudo nmcli connection delete "<old-SSID>".

Tip — do the network change before you unplug Ethernet. If you SSH in over Ethernet and switch Wi-Fi, the Ethernet session stays up, so a failed Wi-Fi join does not lock you out. Confirm Wi-Fi works, then unplug the cable.

## 10.3 — Finding the Jetson's IP the easy way (mDNS)

If the Jetson's hostname is known (say it is devconph), you can often reach it by name without any IP, because Ubuntu advertises itself over mDNS. Try this before scanning:

ping devconph.local

ssh devconph@devconph.local

macOS and most Linux resolve .local out of the box. Windows 10/11 resolve it too via the built-in mDNS. When .local does not resolve — some guest and enterprise networks block mDNS — fall back to a scan (10.4).

## 10.4 — Finding the Jetson's IP by scanning (Windows / WSL with nmap)

When you cannot reach the board by name and the router admin page is not available, scan the local subnet for it. nmap is the reliable cross-platform way. On Windows the cleanest route is to run it inside WSL (Ubuntu), which is also consistent with the SSH client you will use afterwards.

- Install nmap in WSL (Ubuntu):

sudo apt update

sudo apt install -y nmap

(On native Windows instead of WSL, install Nmap from nmap.org, which includes the nmap command for PowerShell. The scan syntax below is identical.)

- Find your own subnet first, so you know what range to scan:

# In WSL:

ip route | grep default        # e.g. default via 192.168.1.1

# Or in Windows PowerShell:

ipconfig                       # read the IPv4 address + subnet

If your laptop is 192.168.1.23, the network is almost certainly 192.168.1.0/24.

- Scan the subnet for live hosts:

nmap -sn 192.168.1.0/24

The -sn flag is a ping sweep — it lists every device that answers, with its IP, and its MAC address plus vendor where it can resolve one. The Jetson's MAC vendor often shows as NVIDIA or the Wi-Fi module maker (e.g. Realtek / Intel), which is usually enough to pick it out of the list.

- Confirm a candidate is really Kai by checking its open ports:

nmap -p 22,8081 192.168.1.42

Kai answers on 22 (SSH) and 8081 (the dashboard). A host with both of those open is almost certainly the Jetson. Once you have the IP, SSH in (10.1) and, if you need to move it to a different Wi-Fi, run the nmcli steps in 10.2.

| Task | OS / shell | Command |
| --- | --- | --- |
| Reach board by name | macOS / Linux / Windows | ssh devconph@devconph.local |
| Ping sweep the subnet | Windows (WSL) / Linux / macOS | nmap -sn 192.168.1.0/24 |
| Confirm it's Kai | any with nmap | nmap -p 22,8081 <candidate-ip> |
| SSH in | any | ssh devconph@<jetson-ip> |
| Change Wi-Fi (on Jetson) | Ubuntu / L4T | sudo nmcli device wifi connect "<SSID>" password "<pw>" |
| Read new IP (on Jetson) | Ubuntu / L4T | hostname -I |

Note: this sheet stays hardware- and connectivity-only. Operating Kai, the live settings and the software side of these fixes are documented in docs/operating.md and docs/faq.md.